1924 – 28 Studebaker Big 6

# Specifications

Motor Maintenance Data Book

And Flat Rate Manual

1928 Edition

Manual Data								Personal Notes*

## Front Axle

Camber (in.)				2

Caster (deg.)				0

Gather (in.)				3/8

King Pin Angle (deg.)			4 ¾

Wheel Bearing Number (inner)	3381-3331

Wheel Bearing Type (inner)		Roller

Wheel Bearing Number (outer)	2681-2631

Wheel Bearing Type (outer)		Roller

Axle Bush ID & OD (in.)		13/16 X 1 1/16

Axle Bush Length (in.)		1 3/8 & 1 1/16

Tie Rod ID & OD (in.)			5/8 X 7/8

Tie Rod Bushing Length (in.)		7/32

Make of Axle				Studebaker

## Rear Axle

Make of Axle				Studebaker

Type of Axle				½ F				½ Floating

Gear Ratio				4.08

Factory Part Number			32784

Bevel or Worm Drive			SB				Spiral Bevel

Type of Bearing			Roller

Factory Part Number			43181

Factory Part Number Pinion		107204

## Brakes – Rear External

Lining Thickness (in.)			3/16

Lining Width (in.)			2 ½

Lining Length (in.)			150

## Brakes – Transmission Brake

Lining Thickness (in.)			3/16

Lining Width (in.)			3

Lining Length (in.)			25

Front Wheel Brake Drum #		114595

Rear Wheel Brake Drum #		114596

## Body

Running Board # - Right		112482

Running Board # - Left		112483

Body (contd.)

Front Fender # - Right		109831

Front Fender # - Left			109832

Rear Fender # - Right			215290

Rear Fender # - Left			215291

Running board Splasher # - Right	109655

Running board Splasher # - Left	109656

## Clutch

I-D					8 ¾

O-D					12

Thickness				5/32

No. Per Car				2

Make of Throwout Brg.		Studebaker

Factory Part Number			109146

## Connecting Rod Bearing

Make of Engine			Studebaker

Type of Bearing			Pou.				Poured

Factory Part Number			109030

Rod Assembly Removed		Above

## Piston & Pin

Make of Engine 			Studebaker

Material of Piston			CI				Cast Iron

Factory Piston Number		112705

Factory Piston Pin Number		106499

## Piston Rings

Number of Rings per Piston		4

Width of Rings (in.)			3/16

Factory Part Number			39401

## Radiator

Make					McCord

Type of Core				VT				Vertical Tube

Capacity (gal.)				4 ¾

Type of Cooling			P				Pump

Factory Part Number – Core		112516

Factory Part Number – Shell	112517

## Crankcase

Make of Engine			Studebaker

Upper Case Material			Ir.				Iron

Lower Case Mateial			PS				Pressed Steel

## Main Bearings

Number of Main Bearings		4

Offset In.				No

Front Bearing Number		112066

Middle Bearing Number		112068

Rear Bearing Number			112069

## Valves

Make of Engine			Studebaker

Valve Arrangement			L

Valve Head Material			A St.				Alloy Steel

Tappet Clearance Inlet (in.)		0.006 H

Tappet Clearance Outlet (in.)		0.006 H

Inlet Opens				7 ½ A

Exhaust Closes			11 A

Factory Part Number Inlet		32090

Factory Part Number Outlet		32090

Factory Part Number Valve Guide	16727 Inlet

Factory Part Number Valve Guide	16727 Outlet

## Cylinders

Make of Engine			Studebaker

Number of Cylinders			6

Bore & Stroke (in.)			3 7/8 X 5

## Headlamps

Lens Diameter (in.)			9 ½

2 – Filament Bulb			Yes				Mine, Single Filament

## Generator

Charging Rate Hot			12

Charging Rate Cold			14

Armature Speed			2000

Make					Wagner or Remy		Mine, Wagner – Type S-508

## Starting Motor

Battery to Motor Cable Part No.	109707

Battery Amp Hrs.			111

Make					Wagner or Remy		Mine, Wagner – Type S-503

Distributor								I added this category head

Breaker Point Gap (in.)		0.025

Spark Advance Degree		DC

Make					----				Mine, Wagner K814, SN 5D

## Front Springs

Factory Part Number			110923

Length (in.)				38

Factory Part Number Bushing	39569

Factory Part Number Bolt		109099

## Rear Springs

Factory Part Number			112215

Length	(in.)				56

Factory Part Number Bushing	109120D

Factory Part Number Bolt		109121

Chassis Lubrication System		Zerk				Mine, Alemite

## Carburetor

Make					Ball				Mine, Penberthy Ball & Ball

SV-29

Size					1 ½

Fuel Tank (gal.)			19

## Oil Pump

Pump Type				Ge				Gear

Oil Pressure (psi.)			10

At Miles Per Hour			25

## Propeller Shaft

Front Universal Type			Metal

Rear Universal Type			Metal

Type of Drive				H				(?)

## Steering Gear

Make					Studebaker

Type					W & W				Worm and Wheel

Steering Column Upper Bushing #	124052

## Transmission

Make					Studebaker

Clutch Gear Shaft or Brg. (?) No.	123061

## Tires

Tire Size				32 X 6.75			Mine, Solid wheel,

BFG Silvertown

Power Expr. 6.50 X 20

Rim Size				19 X 5

Make of Rim				Budd

## Brake Drums

Diameter Drum Front Wheel (in.)	16

Diameter Drum Rear Wheel (in.)	16 5/16

* Personal Notes – These include translations and comments relative to “Mine”. Any comment starting with “Mine” relates to my 1925 Studebaker Big 6, Engine S/N EP 8358, Chassis S/N 2068136. This data was compiled by Stephen P. Upham III on 11-2-00. Any comments and / or corrections are welcome. Email address spu@spa.net